Annexure 01

CEYLON SHIPPING CORPORATION LIMITED

DRYDOCK SPECIFICATION

(Class Renewal survey No. 2 at 10th year)

FOR

## M.V. CEYLON BREEZE

(Built in 2016 Jan)

DRYDOCK DUE

January  2026

OWNER: CEYLON SHIPPING CORPORATION LIMITED

M.V. CEYLON BREEZE

MAIN CONTENTS:-                 PAGE NO:

1.	PARTICULARS OF VESSEL.			2-4

2.	PREAMBLE TO SPECIFICATIONS.		5-38

3. 	Ship side valve spec				39-40

4.   Job list						41

ATTACHMENTS:

1.  DRAWINGS:- Annexure 03

a.	DOCKING PLAN

b.	GENERAL ARRANGEMENT

c.	MIDSHIP SECTION

d.	SHELL EXPANSION

e.	PROPELLER specs

f. 	Rudder calculation

g. 	Bottom Plug arrangements

- SCHEDULED JOBS-Annexure 02

- CB/DD2/DK  001- 050

- CB/DD2/EN  001- 038

- CB/DD2/EL  001- 016

- SHIP SIDE  VALVE SPECS- PAGE 39-40

WE REQUEST YOU TO QUOTE IN US DOLLARS AND FILL IN BLANKS IN ENCLOSED SHEETS ONLY. FOR GUIDANCE GIVE EXCHANGE RATE 1 USD =________ USED FOR THIS QUOTE.THIS WILL LEAD TO FASTER COST COMPARISON WITH MORE UNIFORMITY WITH INCOMING QUOTATIONS.YOUR QUOTATION MAY NOT BE CONSIDERED IF ABOVE PROCEDURE NOT COMPLIED WITH.

DOCK REPAIRS SPECIFICATION

VESSEL’S PARTICULARS

NAME					:	MV CEYLON BREEZE

MANAGER				: 	CEYLON SHIPPING CORPORATION

27, MICH Building

Sir Razik Fareed Mawatha

Colombo 00100

Sri Lanka

TEL :94 112422807

FAX :94 112449486

E-MAIL:technical@cscl.lk

P.O.R			            :	Colombo

CALL SIGN				  :	4RDW

Official NO			  :  1361

FLAG					  :	Sri Lanka

LOA X BMLD X DepthMLD	  :	199.826M X 32.249M X 18.5M

Draft(S)				  :  13.299

DIST BET KEEL AND

HIGHEST POINT 			  :  48.40M

GROSS TONNAGE            :	36432.00 TONS.

DEADWEIGHT TON             :  63323.9 MT (SUMMER)

TPC					  : 62.3MT

YEAR BUILT			       : 	2016

KEEL LAID				  : 18.12.2014

LAUNCHED				  : 31.08.2015

Delivered				  : 30.01.1016

BUILDER				:   AVIC WEIHAI SHIP YARD CO LTD. China

KIND OF SHIP/ Hull No	:   Bulk Carrier/AVIC398

CLASSIFICATION/ CLASS NO.:    DNV-GL

IMO NUMBER/OFFICIAL NO.	:    9734563

MAIN ENGINE			:    Man B&W 5S 60ME-C8.2 Tier II

AUXILIARY ENGINE		:    DAIHATSU 6/DK20e. 3 SET.

BOILER - TYPE X SET		:	SAACKE CMB- 1.5+0.96, 120KG/hr.

ELECTRIC CURRENT	     	:	440 V / 230 V 60HZ 3PHASE + 24 V

DOMESTIC SUPPLY		     :	230 V

CARGO HOLD CAPACITY		: (GRAIN 78763.99 / BALE 73680 CBM) 100%

| #1 CARGO HOLD | 19.68 m x 18.26 m /13,957.79 Cm |
| --- | --- |
| #2 CARGO HOLD | 22.96 m x 18.26 m /17,658.30 Cm |
| #3 CARGO HOLD | 22.96 m x 18.26 m /15,352.99 Cm |
| #4 CARGO HOLD | 22.96 m x 18.26 m /15,837.60 Cm |
| #5 CARGO HOLD | 22.96 m x 18.26 m /15,962.36 Cm |

BALLAST TANK CAPACITY    : (17500 IN M3)

| FPT | 1372.94 |
| --- | --- |
| NO.1 WBT (P) | 1622.85 |
| NO.1 WBT (S) | 1639.64 |
| NO.2 WBT (P) | 1834.25 |
| NO.2 WBT (S) | 1834.25 |
| NO.3 WBT (P) | 1840.37 |
| NO.3 WBT (S) | 1840.37 |
| NO.4 WBT (P) | 1117.32 |
| NO.4 WBT (S) | 1117.32 |
| NO.5 WBT (P) | 1175.52 |
| NO.5 WBT (S) | 1175.52 |
| AP Tk | 1136.15 |
| No.3 CH | 15352.99 |

FRESH WATER CAPACITY	85%	:507M3

FUEL OIL CAPACITY	85%	:1643 M3

MDO CAPACITY	     85%	:180.01M3

MAIN ENGINE Control system.

MAKE:KONSBERG

TYPE-:K-chief 600

AUX. ENGINE GOVERNOR-3NOS.

AUX ENGINE MAKE:DAIHATSU

TYPE:6 DK 20e

GOVRNOR:

Make:ZEXEL

TYPE:- RHD 6

Intermediate Shaft / Bearing & Tail shaft:  survey /withdrawal.

STERN TUBE  Seals : Wartsila 
Seal type         : STERN GUARD OLS4-P / OLS2-P

SEAL RING DWG. NO.: SS003879 & SS3880

ONE Intermediate Shaft Bearing — 445 mm Dia x 650 mm L,  
Tail shaft – LENGTH 5050 MM, DIA 420 MM

Tail shaft LINER : 430 mm Fwd/ 3505 mm aft
Intermediate shaft : 350	 mm dia x 6400 mm L

PROPELLER AND PROPELLER SHAFT

(PROPELLER)

TYPE: 5 BLADES SOLID NBS TYPE,SKEWED (320) Right Hand

MATERIAL: NI-AL-BRONNZE Cu3

SIZE: DIA=6700 MM

PITCH: 5804.06MM

MAKER: NAKASHIMA PROPELLER CO.,LTD

Weight : 20395Kg

(PROPELLER SHAFT)

SIZE: D=420 MM X L=5,050MM X 1 SET

MATERIAL:KSF60

MAKER: SHIN KURUSHIMA DOCKYARD CO., LTD.

(INTERMEDIATE SHAFT)

SIZE:DIA=350MM X L=6,400 X 1 SET

MATERIAL :KSF65

MAKER: SHIN KURUSHIMA DOCKYARD CO., LTD

General description:  The vessel is a single hull bulk carrier with 5 cargo hold and 4 deck cranes.

PREAMBLE OF THE SPECIFICATIONS

1.	The term "OWNER" as used in the preamble to the specifications shall be deemed to mean "Ceylon Shipping Corporation”.

The terms "CONTRACTOR", "YARD", and "SHIPYARD" are to be interpreted to mean the prime contractor to whom the invitation to bid is directed.

The term "SUBCONTRACTOR" as used herein shall be understood to mean any firm other than the contractor engaged to perform any service to the subject vessel whether hired by the contractor or the owner.

2.	Tenders are to be addressed and delivered to the “tenders-drydock@cscl.lk”

CEYLON SHIPPING CORPORATION

27, MICH Building

Sir Razik Fareed Mawatha

Colombo 00100

Sri Lanka

Attn.: TECHNICAL DEPARTMENT

3.	Tenders must specify the total number of consecutive calendar days required by the contractor to complete the repairs itemized in the specification. The vessel will be opened for docking  during the time period from 15th January  to 15th February 2026.

4.	Shipyard is to furnish individual costs in their tender for all items contained in the “Scheduled job” specification.

5.	The vessel shall be considered tendered for repairs when she is off the contractor's shipyard ready to be secured at the wharf. Vessel will be accepted afloat by the owner upon completion of all repairs, renewals and replacements and after all contractor's equipment, tools and appliances, also dirt debris and old material have been removed within the allowed dry-docking period by the contractor.

6.	For the time the vessel is tendered to the contractor and until she is redelivered to the owner, upon completion of the work, it shall be the responsibility of the contractor to comply with all specifications, equipment, applicable and regulatory to classification bodies that apply to all repairs, renewals, alterations, use of fire, burning welding, riveting and other such methods used in carrying our such work, whether or not specifically stated in the owner's specifications.

7.	Contractor to clean and gas free and to obtain gas free certificate for respective spaces in which hot work will be performed.

8.	Any tender which includes modifications, alterations and / or special clauses May be rejected by the Owner unless agreed in advance.

9.	Vessel shall be dry-docked immediately after arrival at the contractor's shipyard, unless stipulated otherwise by the owner or mutually agreed between the contractor and the owner.

10.	Where dry-docking is required for regular periodic overhaul, contractor shall allow sufficient time in dock to properly apply paint or carry out other work as called for in the specifications.  Additional time in dock for work not mentioned in the specifications shall be mutually agreed upon between the contractor and the owner.  In connection with additional days in dock, should the contractor request an interruption of dry-docking, any costs/LOH???? relative to undocking, shifting, mooring, re docking, interruption of repairs, etc., are to be borne by the contractor.  The contractor is to state number of days subject vessel is to be held on dry-dock, as well as the cost thereof the carry out the work outlined in the specifications.

11.	All scrap in connection with the work specifications, shall be inspected and decided by the owner’s representative, before hand over them to the shipyard. No extra cost will be applied for any Scrap/Sludge/Oily water/garbage removal.

12.	When any work performed by the contractor requires the opening of machinery, piping, heat exchangers, fittings, etc., which are directly or indirectly connected to a source of possible leakage into the vessel, such as sea chests, overboard discharge valves, crossovers, connections to the shell of the vessel, tanks or pipelines containing liquid, or any other source of leakage, it shall be the responsibility of the shipyard to fit blanks as necessary to eliminate the possibility of leakage into any space or equipment of the vessel and/or remove the liquids ashore at his own expenses.

Upon completion of any such repairs, all blank fitted shall be removed and after reassembly, these items shall proven tight. These precautions shall be taken as necessary to prevent any damage to the vessel whether it is afloat or in dry-dock.

Cost for above mentioned blanks is to be included when cost for job is quoted.

13.	In the event of any dispute between the parties arising under or pursuant to the repair contract, such dispute shall be referred to the decision of a single  arbitrator in Sri Lanka in accordance with the Sri Lanka Arbitration  act no.11 of 1995 with English law.

1.	GENERAL

1.1	The specifications attached hereto describe repairs, renewals and/or alterations to be effected to the above named vessel and are to cover the furnishing of all necessary labor, machinery, materials, equipment and spare parts required for the contractor to complete the items of work outlined herein (expect those items of machinery, equipment and spare parts specified to be furnished by the owner) in accordance with this specification.

1.2	All costs including but not limited to removing and refitting of parts obstructing access, cleaning, de-rusting, reconnecting and testing components, material, machinery and outfitting, temporary lighting, ventilation, staging, rigging, cranage, creation and assembling aids of any kind, removal and transportation of items to and from the workshop if necessary, reinstallation and cleaning up of work area and areas affected by the work, are to be for contractor's account and are to be taken into consideration when bidding/ quoting for the job.

Where specifications require the opening of machinery, piping, fittings, etc., it is intended that the cost quoted includes the removal and refitting of and renewal where found necessary- of all disturbed gaskets, packing, fastenings, securings clamps etc and the repair of renewal of damaged insulation.

All costs including but not limited to fitting, connecting, grit-blasting, painting and testing new components, material, machinery and outfitting, removal of liquids and deposits from engine room and pump room, holds, upper/lower stools and other spaces in which contractor performed work are to be for contractor's account and are to be taken into consideration when bidding/ quoting for the job.

1.3	Whenever the word "OVERHAUL" is used in the specifications and no other specific requirements are mentioned, contractor shall allow for disconnecting from existing fittings, removal from position where necessary, dismantling, removal to workshop, opening-up, cleaning, de -rusting and painting.  Elsewhere, arranging for inspection, reassembling in good working order, using new bolts, nuts, joints and packings, fastenings, clamps, etc reinstalling on board as originally fitted and  testing to be included and taken into consideration when quoting / bidding for the job.

1.4	Any structural parts specified to be renewed which can be restored to their original condition by fairing, etc. and also any parts specified to be removed for fairing which can be faired in place to the satisfaction of the surveyor and the owner's representatives will be accepted.  On the other hand, any parts found broken or which are broken in removal of fairing are to be renewed by the contractor at his expenses.

1.5	All new work (piping, steel plates, brackets, etc.), alternations, repairs, modifications, etc. in cargo holds, on decks or other areas which are to be coated (or are being coated during this dry-docking) are to be suitably prepared, and after approval by the owner's representatives, coated with the same type coatings (approved by the owner's representatives) as have been or are being applied to the balance of these areas; it is being the intention of the owner that all steel surfaces in such areas be coated.

All adjacent coated areas disturbed during the fore-going work shall be similarly prepared and coated.

1.6	All workmanship and/or material is to be of the best quality and any dispute or difference of opinion as to the interpretation of these specifications or any part thereof regarding the quality of material and/or workmanship shall be left to the decision of the owners, whose decision shall be final and binding to both parties. Owner reserve the right to supply materials, spares or goods if required for any work / repair/ service which to be carried out by the contractor.

1.7	The owners reserve the rights to correct errors or omissions in, or to make deductions from, or additions to the specifications. However, the increased or decreased costs, if any and time of completion herein specified shall be adjusted accordingly. Where work specifications increase / decrease, the unit [discounted] rate quoted for the job, will be taken in calculating the final price pro-rata.

1.8	Any particulars of the work involved specified herein are given only for the guidance of the contractor, who will be held responsible for the securing of all necessary dimensions and details, the intent of these specifications being to restore the vessel to first class sailing condition.

1.9	If the contractor considers, it is necessary to use the ship's tools, machinery, etc. this will be granted. However, the contractor will restore all of them to their original condition. under normal circumstances, all jobs are to be undertaken using the contractor’s tools & lifting appliances.

1.10	Tenders must specify the time required by the contractor in consecutive calendar days to complete the repairs, renewals, alterations and/or conversions set forth in the specifications attached hereto, and itemized fixed rates for all services to be furnished by the contractor.

1.11	Owners reserve the rights to perform normal overhauls on deck and in the engine room by using their own crew or sub contractor while vessel is on contractor's premises. This may involve hot work, and relevant hot work permit will be applied for by the ship staff and will be granted by the yard’s safety personnel accordingly.

1.12 As well Owners reserve the rights to invite OEM or other service providers or their representatives to perform Service, Repair, Supply of any equipment/Machinery/spares onboard the vessel. The contractor shall permit employees of such subcontractors access to the shipyard and the vessel for such purpose. There will be no surcharge applicable/ levied by the shipyard for the above, either to the Owner or to the subcontractors.

1.13	Before arrival of the vessel at the contractor's shipyard, no work in connection with any of the specifications to be begun without the permission of the owners who also have the rights to cancel any of the specified items against respective credit. Scope of work to be undertaken is to be discussed between the Owner’s representative and the contractor’s SRM prior to commencement of any repairs.

Following arrival of the vessel at the shipyard, no work exceeding the scope of the specifications to be commenced without the written confirmation of the owner's representatives. Work orders or work completion reports signed by any person other than MASTER / CHIEF ENGR and countersigned by the attending owner’s superintendent, will not be considered.

1.14	There shall be no departure from these specifications unless directed, in writing, by the owner or the owner's authorized representative.  The owner has the right during the performance of this work to make alterations before the particular work is commenced.

1.15	In the event the contractor desires to subcontract any work or services covered by the specifications or additional work agreed upon, the contractor shall give notice of such intention to the owner, together with the name of the subcontractor who will perform the work or service or engage a subcontractor acceptable to the owner.

1.16	With regard to owner-furnished material and equipment, the shipyard is to transport all such items to the point of installation aboard the vessel, whether stored aboard the vessel or delivered to the shipyard.  In the latter case, the shipyard is to accept delivery of any such material and store if necessary until the time of installation. No extra surcharge will be levied for this service.

Any unused material, as designated by the owner's representative is to be removed ashore, and held for the owner's disposition.

Any costs in connection with the foregoing are to be included in the bid price of any such items in the specifications.

1.17	It is the intent of the specifications to restore the vessel to the same good condition as existed prior to the repairs being required and to ensure that high quality standard of workmanship shall be maintained with reference to renewals and new work and that all materials shall be of the best quality throughout satisfactory to the owner.  All materials requiring a test shall be tested in accordance with paragraph 6 of the preamble and must meet all requirements thereof.  All charges and costs for tests to be borne by the contractor.

1.18	No overtime shall be worked for the owner's account unless the owner's representative shall approve the items to be worked on, the maximum number of overtime hours, and the department authorized to perform the overtime and the resultant cost as requested by the owner's representative. Any overtime incurred to meet vessels schedule not for owners account.

1.19	No extra compensation or added time for any additions, repairs, or alterations will be allowed without approval by the owner's representative, and confirmation in writing by owner’s representative, for such additions, or alterations before same are commenced. Prior commencement of the work and within 24 hours of the additional work being requested, the contractor shall submit an estimated cost for the additional job scope, and the final cost of the additions, repairs or alterations properly authorized as indicated above shall be arrived at, using bid prices where they exist or by estimating cost during current rates where they do not.

1.20	All charges in connection with towage and pilotage services for entering and leaving the contractor's shipyard will be for the owner.  All towage and pilotage services and the charges thereof and other services and charges in connection with shifting the vessel, mooring, wharfage, line handling.  Labor and laborers for shifting, shall be borne by the contractor except where stated otherwise in the specifications that follow, advise costs for tugs pilots per port rules.

1.21	Wherein the specifications refer to one (1) unit to be repaired, and upon investigation it is determined that a second (or more) duplicated unit(s) require similar repairs/ renewals, the price quoted for the first unit shall apply to each additional unit.  The cost of additional unit shall take into consideration the cost of removals and replacements that are common to the first unit.  Likewise, where a fewer number of units are dealt with than stated in the specification; the cost shall be reduced in proportion to the bid price.

1.22	After completion of all work, contractor has to supply report in duplicate containing all measurements taken and drawings of the modifications performed, which shall be presented to owner's representative not later than upon agreement of amount of the final yard invoice.

1.23	Contractor to confirm that if vessel is in dry-dock over weekend (Saturday, Sunday) or over local holidays, normal work will continue as per the job scope, and if no work authorized by owner's representative is carried out, then no dry-dock charges will be levied.

1.24	Any holidays during expected repair period to be clearly indicated along with the quotes.

1.25	The contractor will provide a bar chart along with the quote, 	indicating the job schedule and clearly marking the critical 	jobs that control the period of repair. The yard , during 	the docking, will indicate on the following day, the updated 	estimate of expenses incurred for the repairs/ docking if same 	is not within 5% of the initial projected costs.

1.26 It is mutually agreed that payment terms are proposesed  as follows:

40 percent of the final agreed amount on redelivery.

50 percent of the final agreed amount after 2  months of redelivery .

10 percent of the final agreed amount after one  year of redelivery.

1.27 Proforma invoice to be mailed to the Ceylon Shipping Corporation for approval.

1.28 Contractor will present draft invoice to the owner’s 	representative, Two Banking  days prior to redelivery of the vessel, 	based on the work scope being undertaken until then. Final 	draft invoice will be submitted to the owner’s representative 	at least 24 hours prior to redelivery of the vessel.

1.29 Penalty clause: It is mutually agreed that if the work 	contracted is not completed within the time stipulated in the  Agreement, the 	contractor shall pay as damages to the owners, for every day 	or part thereof in excess of the initial quoted time, the sum 	of US $ 10,000.

1.30	In case of any damage to vessel due to circumstances beyond the vessel’s control and/or due to negligence by the yard and/or its personnel, yard will be liable for all costs towards repairs to vessel, off-hire and loss of hire costs and all associated costs related to the damage repair with no pre-set limit.

1.31 The contract may be canceled / voided at the sole discretion / election of the owners, if the contractor, due to overload or other reasons, is unable to begin work on arrival of the vessel at the contractor’s premises/ shipyard, or unable to undertake any part of the repairs outlined in the specifications within the contract period quoted by the contractor in their bid response. Owner’s costs due to such cancellation, will be recoverable from the contractor.

1.32 The owner is at liberty to use his crew to carryout repair work during the docking either in the dry dock or at the lay berth always without hindrance to the yard’s work plan and always following the yard’s and local safety procedures. The owner’s representative will inform the SRM at least 12 hrs prior to the commencement of the work by ship staff.

1.33 If the yard submits their standard terms and condition along with the quote, then the yard should specify clearly separately on a letter any item that is in conflict of what the owners require. Failing this the terms and condition said here will be deemed final.

1.34 While the superintendent from Ceylon Shipping Corporation on behalf of Owners, will be fully responsible for placing the order, monitoring the docking, confirming additional orders during the course of docking and negotiating the final invoice, the full responsibility of payment of the yard invoice will be the sole responsibility of the Vessel’s owners - Ceylon Shipping Corporation Ltd.

  - Use of Asbestos is forbidden on board the vessel. The yard to confirm in writing and ensure that all repairs are carried out without use of Asbestos packing and lagging. No form of Asbestos is to brought on board during the period of repairs. Yard to provide proof in form of certificates to the vessel’s master prior departure listing all NON Steel, wood and Plastic materials used in carrying out repairs and confirm that these materials are Asbestos Free.

  - Yard will, on vessel’s arrival take over the vessel’s security from the SSO and hand over the security back to SSO prior departure. The take over and hand over is to be fully documented.

  - During the course of repairs, yard is obliged to consult with the Owner’s superintendent prior arranging Class Surveyor’s visit. Cost of Surveyor’s visit and waiting time for arrangement without approval of Owner’s superintendent will be for yard’s account.

- Itemized Prices

2.1	Cleaning And Painting Work:-

Hull Cleaning (per/1 sq.m)

a.	Low pressure fresh water wash             USD_______________

b.	Solvent washing of oily places            USD_______________

c.	Hard scraping off loose paint and rust.   USD_______________

d.	Mechanical wire brushing.                 USD_______________

e. Power tooling						 USD_______________

f.	Washing and cleaning with high

pressure water jet (min 200 bar)          USD_______________

g.	Grit blasting to SA - 2                   USD_______________

Spot blasting for above                   USD_______________

h.	Grit blasting to SA - 2.5                 USD_______________

Spot blasting for above                   USD_______________

i.	Grit blasting to SA - 1.5                 USD_______________

Spot blasting for above                   USD_______________

j.	Grit sweeping.                            USD_______________

k.	Surcharge (pcnt) for Grit blasting,       USD_______________

sweeping in ballast

tanks.

l.	Price for necessary staging per Cu m

for above.                                USD_______________

Hull Painting With Owners Paint (Per 1 SQ M)

a.	First and subsequent anticorrosive coats. USD________________

b.	First and subsequent antifouling

SPC coats.   						 USD________________

c.	Touching up anticorrosive.                USD________________

d.	Touching up antifouling SPC               USD________________

e.	Surcharge (pcnt) for painting in tanks.   USD________________

f.	Price for necessary staging for above

Cu.m.                                     USD_________________

g.	Painting draft marks, ships names, por,

plimsoll marks, bulbous bow marks, tank

division marks, tug 	marks etc (two coats) USD______________

h. Touch up on spots (Derusting/primer/final coat) USD ________

( Quote for a spot rate)

2.2 STEEL PLATE RENEWAL:-

| TOTAL STEEL RENEWED IN TONS. | PRICE/KG. | SHAPED SINGLE CURVATURE. | SHAPED DOUBLE CURVATURE. |
| --- | --- | --- | --- |
| 2 TO 10 |  |  |  |
| 10 TO 20 |  |  |  |
| 20 TO 50 |  |  |  |
| 50 TO 100 |  |  |  |
| 100 TO 200 |  |  |  |
| ABOVE 200 |  |  |  |

Note:-

a.	Incase of renewal of less than 2 tons, add under said percentage to prices stated for 2 - 10 tons.

Steel                      Percentage

up to 100kg              _____________

100kg to 500kg           _____________

500kg to 2000kg          _____________

b.	Above prices refer to ordinary ship building steel and to welded construction.

c.	Unit steel weight is assumed 8kg/dm3.

d.	Weight of steel to be calculated on sizes actually fitted, without allowing for wastage, and quoted price to apply for calculation of cost of work, whatever be the size and extent of the work.

STEEL PLATE RENEWAL:

| TOTAL STEEL RENEWED IN TONS. | PRICE/KG. | SHAPED SINGLE CURVATURE. |
| --- | --- | --- |
| 2 TO 10 |  |  |
| 10 TO 20 |  |  |
| 20 TO 50 |  |  |
| 50 TO 100 |  |  |
| 100 TO 200 |  |  |
| ABOVE 200 |  |  |

Note:-

a.	Incase of renewal of less than 2 tons, add under said percentage to prices stated for 2 - 10 tons.

Steel                      Percentage

up to 100kg              _____________

100kg to 500kg           _____________

500kg to 2000kg          _____________

b.	Unit steel weight is assumed 8kg/dm3.

c.	Weight of steel to be calculated on sizes actually fitted, without allowing for wastage, and quoted price to apply for calculation of cost of work, whatever be the size and extent of the work.

STEEL PLATE RENEWAL:

| TOTAL STEEL RENEWED IN TONS. | PRICE/KG. | SHAPED SINGLE CURVATURE. |
| --- | --- | --- |
| 2 TO 10 |  |  |
| 10 TO 20 |  |  |
| 20 TO 50 |  |  |
| 50 TO 100 |  |  |
| 100 TO 200 |  |  |
| ABOVE 200 |  |  |

Note:-

a.	Incase of renewal of less than 2 tons, add under said percentage to prices stated for 2 - 10 tons.

Steel                      Percentage

up to 100kg              _____________

100kg to 500kg           _____________

500kg to 2000kg          _____________

b.	Unit steel weight is assumed 8kg/dm3.

c.	Weight of steel to be calculated on sizes actually fitted, without allowing for wastage, and quoted price to apply for calculation of cost of work, whatever be the size and extent of the work.

2.3	Internal Steel Work:-

a.	Tank top plates, straight bulkheads

surcharge pcnt.                           _________________

b.	Tank margin plates, casing plates,

bulkwark plates, shell frames, brackets,

stringer, bulwark caprail and stanchions

surcharge pcnt.                            _________________

c.	For corrugated bulkheads and joggle plates

surcharge pcnt.                            __________________

2.4.1 	Fairing On Spot:-

Quote per 1 sq m.

a.	Shell plates.              USD________________

b.	Deck and internals.                          USD_______________

2.4.2 	Off, Fairing And Refitting:-

Quote per 1 sq.m.

a.	Shell plates.                         USD_________________

b.	Deck and internals.                   USD_________________

  - Pipe Repairs:-

2.5.1

LT Cooler???

- Supplying and fitting steel pipe (SA 106 GR B sch 40 or equivalent) using existing flanges according to Column A.

b.	Steel bends according to Column B.

d.	Steel flanges according to Column C.

e.	Surcharge in percentage of prices for steel

material for renewal of copper pipes,

bends and flanges                      USD               .

| PIPE DIAMETER IN MM | A PER METER | B PER BEND | C PER FLANGE | DECK AND BHD PENETRATION PCS. |
| --- | --- | --- | --- | --- |
| UP TO 12MM |  |  |  |  |
| 12 - 25 MM |  |  |  |  |
| 25 - 50MM |  |  |  |  |
| 50 - 75MM |  |  |  |  |
| 75 - 1OOMM |  |  |  |  |
| 100 - 125MM |  |  |  |  |
| 125 - 200MM |  |  |  |  |
| 150 - 200MM |  |  |  |  |
| 200 - 250MM |  |  |  |  |
| 250 - 300MM |  |  |  |  |

NOTES:-

a.	Staging, clamping, testing and painting included.

b.	Additional percentage surcharge for

galvanized steel.

c.	Additional percentage surcharge for

pipes in	Engine Room.

d.	Additional percentage surcharge for

renewal in pump room, deep tanks and

cargo tanks.

e.	Additional percentage surcharge for

renewal in double bottom, pipe tunnels

& cofferdams.

f.	Additional percentage surcharge for

thick walled schedule 80 pipes.

- Additional percentage surcharge for

Yorcalbro 	pipes.

h. Additional percentage surcharge for

Stainless steel SUS 304 pipes       ___________________

i. Additional percentage surcharge for

Stainless steel SUS 316L pipes      ___________________

j. Additional percentage surcharge for

stainless steel K SUS 316 L pipe    ___________________

k. Additional percentage surcharge for

stainless steel K SUS 304 pipe      ___________________

l. Additional percentage surcharge for

STPG- Sch 40 pipe                   ___________________

2.6	Quote for Renewal of the following:

a.	Bulkwarks / Rate per kg            USD                /kg.

b.	Checkered plate per kg             USD                /kg.- Ship staff

2.7	Weld Seams Cleaning (includes staging):

a.	Per meter blasting of weld seams    USD                /M.

b.	Per meter rotary brushing of        USD                /M.

weld seams

c.	Per meter price for chipping, V-out and

rewelding both sides of seams/butts

(8-20mmthick)                        USD               /M.

2.8	Manholes (double bottom, topside tanks):

a.	opening and closing including renewal

of gasket per pc.                   USD               /pc.

b.	Renewal of SS studs/nuts and

washer per pc.                      USD               /pc.

2.9	Renewal of owner supplied packing in

Mcgregor type cargo hatch cover,

including grit blasting to SA1

and priming. Price per running mtr  USD                /M.

3.	Repair Period General Services

All items to be quoted. Sevices to be utilized will be decided as per the requirements of the vessel and the facilities provided by the yard.

3.1	Shore Electric Power

Kind of Power source:-

440    V AC,   3    Phases,  60    HZ.

400    A,    stabilized

a.	Price per time connection /disconnection USD        /time.

b.	Price per KW-HR			 	  	USD      /kw-hr.

Note:-

Readings on contractor's power meter to be confirmed /recorded/ signed  before and after use of shore power by ship's electrician and contractor's electrician jointly.

3.2	Cooling Sea Water

Cooling sea water to be supplied to the ship's plants as listed below and as directed when vessel in docks. Cooling sea water to be supplied continuously with proper inlet pressure.

Quote price per 24 hrs in maintaining service for the following:

a.	For provision refrigeration plant   USD             /24hrs

b.	For air con refrigerating plant     USD             /24hrs

c.	For atmospheric condenser cooling   USD             /24hrs

d. For auxiliary engine cooling		 USD            /24hrs

e. for sanitary line                   USD            /24 hrs

Quote cost for connecting and disconnecting once for each.

a.	For provision refrigeration plant  USD               /time

b.	For air conditioning plant         USD               /time

c.	For atmospheric condenser cooling  USD               /time

d. For auxiliary engine cooling	    USD              / time

e. for sanitary line                  USD              / time

3.3	Compressed Air

To provide two compressed air lines of 7kg/cm2 pressure, one to the deck and another to be connected to ship's service air line.

To maintain a continuous and sufficient supply of compressed air to each unit through out the repair duration.

a.	Price per day in service            USD               /day.

b.	Price per time, connection and

disconnection	USD              /time.

3.4	Ballast Water

a.	Price per ton, supply of ballast

sea water                           USD               /ton.

b.	Price per time conn/disconnection   USD              /time.

Please clarify if the charges are made per tank or per        connection used on board. One connection brought on board      can be used for ballasting many tanks.

3.5	Ventilation Fans

a.	Price per time per pc.fan connection

and disconnection                   USD              /time.

b.	Price per unit per day service      USD               /day.

3.6	Temporary Lighting

a.	Price per day service               USD               /day.

b.	Price per time connection

and disconnection                   USD              /time.

Note :Please note that price of temporary lighting for any work undertaken by the contractor, is to be included in the initial quote for the job.

3.7	Tank Cleaning

To clean tanks upon authorization of the Superintendent and remove sludge/mud from their bottoms.

Cost for cleaning ballast /FW tanks

a.	Price per M3 for deep/wing tanks      USD             /M3.

b.	Price per M3 for double bottom tanks  USD             /M3.

c.	Cementing of FW tanks based on capacity

of the tanks                          USD             /MT.

Cost for cleaning bunker tanks

a. Price per M3 for wing tank            USD              /M3.

b.	Price per M3 for double bottom tank   USD             /M3.

c.	Price per M3 removal/disposal of

oily sludge                           USD              /M3.

d.	Price per M3 removal/disposal of mud  USD              /M3.

3.8	Garbage Disposal

a.	Quote price per day, collection       USD              /M3.

3.9	Fresh Water

a.	Price per M3                          USD             /M3.

b.	Price per time, connection and

disconnection	USD           /time.

Note:-

Readings on contractor's flow meter to be confirmed /recorded before and after use by Chief Officer and contractor's personnel jointly.

3.9.1 Bilge Water Disposal

Quote price for disposal of bilge water and accumulations from engine room . Quote to include providing necessary pumps and collection tanks.

a. Price per M3 of bilge accumulations 	USD           / M3

3.10 Shore 	Crane Service-

- Crane for handling ships stores/spares and provision

Price per hour                        USD             /hr.

Note: please advise minimum charge per use if any.

3.11	Telephone Service and Internet Service

Temporary telephone service and internet services to be provided onboard. Any long distance call, including by ship's crew, must not be accepted unless written approval received from owner's superintendent.

To record every outgoing phone call and attach its copy to work done list. Contractor to provide facsimile transmission services to owner's Superintendent.

a.	Price per time, conn and disconnection    USD           /time.

b.	Price per day, phone rent             USD             /day.

- Any requirement from yard as per local rules to keep any minimum no of telephones on board.

3.12	Staging

To furnish robust, safe scaffoldings where necessary to ensure safe access and working condition .

a. Price per M per plank,

hanging staging                          USD          /M/Plank

b.	Price per M3, tower staging              USD           /Cu m

(with Owner supply Scaffolding material)

C.	Price per M3, tower staging              USD           /Cu m

(With yard supply Scaffolding material)

d.	Cherry picker            /No of Cherry pickers.

Cost of Cherry picker				USD per unit

Note:-

Above quotation to include cost for erecting /removing staging. Kindly note that no “minimum” staging will be accepted, and cost for staging will be calculated on that actually required for the job on hand.

3.13	Firewatch

a.	Daily rate for fire watch        USD               /Shift

No of shifts

b.	Necessary number of watchmen per day as per

local/ shipyard regulations         _____________________

c. Any requirement from yard as per local rules to keep any minimum no fire watch on board. _________________

3.14	Fire Line

Price per day for providing contractor’s pressurized fire 	hoses as and where required for fire protection during the 	docking 						USD             / day

a. Any requirement from yard as per local rules to keep any minimum     no fire lines on board or can it be connected to ship’s fire line.

3.15	Gasfree

a.	First gas check by chemist       USD                /visit

b.	Each subsequent checks           USD                /visit

3.16 Alleyway Cover

To furnish adequate cover sheet over the floors listed below and remove, them prior vessel sails out. This is to be put on written authorization of the master.

- Alley way on poop deck

- Alley way on upper deck

- Officer's saloon and smoking room

- Crew's mess room and smoking room   USD                   / M2

Lumpsum cost for alleyway, stairway and accommodation space protection                      USD                  / M2

By Ship Crew –Papers and rubber mat to be supplied

3.17	Scupper Chute

Scupper chute to be fitted to all deck and hull discharge before painting of hull commenced and remove upon completion of same.

No of deck scuppers leading to ship side: 18 nos.

a.	Price per pc including fitting/removal USD             /Pc.

b. Lumpsum charge for fitting and removal of scupper chutes as 	necessary						    USD

3.18	Bottom Plugs

a.	Screwing out and fitting per piece including renewal of lead 	gasket and removing and renewal of the cement over the plugs 							   USD                /Pc.

b.	Renewal of plug per pc            USD                /Pc.

4.	DryDocking

4.1	Mooring and unmooring

The vessel to be delivered at the time dock master embarked on the vessel after arrival at contractor's territorial water and redelivered at disembarkation of dock master upon completion of all repair works.

Tugs and riggers to be supplied to assist vessel on arrival and departure from repair yard and into and out of dry dock.

Quote price per move          USD

Pilotage into and out of dock USD                  / move

Any shifting of vessel between berths for yard convenience to be for yard account. Advise total number of moves anticipated.

If above costs are included in the drydocking cost, kindly mention the same.

4.2	DryDocking

In case of conventional drydock, vessel to be drydocked on soft topped blocks with suitable height for work specified. Contractor to arrange blocks in a configuration which ensures safe and proper weight distribution during the entire period vessel remains in drydock.

For guidance vessel is flat bottom with no rise of floor.

a.	Quote total running calendar days in drydock to

complete specified work, and subsequent

days alongside repair berth to complete all repairs.

In drydock:          days, at wharf          days.

Above mentioned days to be calendar days, including weekends       	and holidays

b.	Docking and undocking, including first

day drydock dues.                   USD

c.	Dockrent per day for subsequent days USD

d.	To advise drydocking condition

*  Max. recommended displacement               MT

approx mean draft                           M.

Guidance: Vessel will be as light as possible.

* Trim by stern            M (max           M)

4.3	Docking Blocks

a.	Keel blocks shifting per pc.       USD

b.	Side blocks shifting per pc.       USD

4.4	Wharfage

a.	Per day wharfage cost during

repair period                     USD               /day.

5.	Hull Cleaning and Painting

Immediately after emptying the dry-dock, the vessel shall be inspected by the contractor, the owner’s representative and the representative of the paint company [as appointed by the owners]. This inspection will be for the purpose of assessing the extent and type of fouling present and checking the dry film thickness and adhesion of the existing coating. Following this inspection, the representative of the paint company will discuss and agree with the owner’s representative the condition found and the recommendations regarding the surface preparation and coating systems. After agreement is reached, a meeting will be held with all parties concerned, to include the shipyard and their sub-contractors if any, the owner’s representative and the representative of the paint company - the purpose being to establish the procedures to be followed in surface preparation, the coating system to be applied, the method of application and the subsequent inspection prior application of further coats of paint. Only after this procedure has been agreed and signed by all parties, the agreed work should commence. The representative of the paint company and the owner’s representative or person nominated by him, will carry out inspection throughout the preparation and coating processes. Should any shortcoming be detected in the agreed procedure, the owner’s representative will take up the matter with the shipyard, and the shortcoming will be rectified by taking appropriate corrective action immediately. All cost/ additional time in rectifying these shortcomings, will be on the contractor’s account.

5.1	Hull cleaning and surface preparation all external hull and bottom plating:

The hull cleaning and surface preparation etc. will be decided after the vessel docks and is examined in consultation with the painting inspector.

Total area of topside, flat bottom, boottop and vertical bottom

BOTTOM (FLAT PART)				: 4958 m2 	 
 V.BOTTOM,BOOTTOP,RUDDER(OUT SIDE)	: 6134 m2	 
 TOP SIDE, BULWARK,BOWCHOCK(O/SIDE)	: 2498 m2

The yard to quote below for reference purpose.

a.	To remove oil spill marks by suitable solvent

Price per m2             		USD                  / m2.

Allow for :  100 m2            USD

b.  Quote cost for cleaning topside, vertical area, flat bottom, propeller and rudder and rudder trunking by fresh water jet min pressure 400bar

[Contractor to state what pressure is available at the yard if the above mentioned pressure is not available].

Price per m2                      USD                 / m2.

Total area : 13590 m2              USD

c.	Quote cost for scraping off Marine growth adhered on hull plate

Price per m2                     USD                 / m2.

Allow for : 200  m2             USD

d.	Rusted spots on topsides, vertical area and flat bottom to be grit blasted to SA 2.0. There will be no ‘minimum area’ charge applicable or acceptable, and charges will be pro rata on actual area of blasting required and agreed upon.

Price per m2                     USD                  / m2.

Allow for:        m2             USD

e.	Entire topside to be grit sweep to remove all existing paint

Price per m2                     USD                  / m2.

Allow for :                      USD

f.	Entire underwater hull from keel up to top of boot top including flat bottom to be grit sweep to remove all existing paint.

Price per  m2                    USD                 / m2.

Allow for :                      USD

Cost to include all staging/ cherry-pickers of cranage necessary for the above jobs, and for any temporary lighting required for any of the work.

Notes:-

Edges of grit blasted area to be feathered for good keying. All blasted areas to be inspected by owner's superintendent after cleaned by dry compressed air. Blasting during night time should be avoided. If this is not possible, owner reserves the right to stop work if adequate lighting is not provided. If work is stopped for the above reason, owners will not accept any extra cost for delay in vessel's docking schedule.

5.2	Coating   - (PAINTS OWNER SUPPLY)

Paint coat thickness to be full fill the requirement of the Class regulation so that the dry docking can be skipped at the next intermediate survey.

All paint to be applied with airless spray unless otherwise agreed with owners superintendent. NO COATING IS TO BE CARRIED OUT IF ANY OTHER VESSEL IN THE SAME DOCK IS BEING BLASTED AND GRIT REACHES THIS MIDNIGHT SUN.

Suitable means to be taken to obtain straight line between colors below and above deep load line to be applied.

To fit scupper chute to the hull discharges directed by owner's superintendent prior hull repainting commenced and ensured no deck drains dripping down over the wet paint area

No of deck scuppers leading to ship side: 12 nos.

US      / PC)     =       USD

All renewed corrosion protective anodes, speed log sensor and echo sounder probe, portholes in way of hull, and other non-paintable areas to be covered up by protective film or sheet during spray painting. Propeller to be protected from splatter during painting. To remove all these protective coatings/ covers upon completion of repainting work.

Cost lump sum for scupper chutes and protection during painting

USD

Coating process must proceed following strictly to the manufacturer's information or instruction given by attending manufacturer's representative. On completion of grit blasting/ sweeping, surface to be washed with low pressure fresh water and given a coat of primer immediately. If surface preparation is spoilt owing to any delay in applying the primer coat, then cost for preparing the surface once again, is to be on contractor’s account.

The areas and the coating sequence mentioned below is for quotation and reference purpose. The exact area, no. of coats etc. will be decided after examining the general condition after docking in consultation with the painting inspector.

CHECKED FROM PAINTING SCHEDULE

5.2.1 	TOP SIDES 			(Total area :2498 m2)

No. of coats  DFT       Product                Price

1 x T/U  30%  150        A/C Primer         USD

1 x T/U  30%  35        A/C Primer          USD

1 x F/C  100% 30        CR Coating          USD

5.2.2 	VERTICAL BOTTOM		(Total area :6134 M2)

No. of coats  DFT       Product                Price

1 x T/U  30%  150        A/C Primer         USD

1 x T/U  30%  100        A/C Primer         USD

1 x F/C  100% 100       Antifouling SPC     USD

1 x F/C  100% 100       Antifouling SPC     USD

5.2.3    BOOT TOP			(Total area :          M2)

No. of coats  DFT       Product                  Price

1 x T/U  30%  150	    A/C Primer		    USD ____________

1 x T/U  30%  100        A/C Primer		    USD ____________

1 x T/U  30%  100        A/C Primer          USD ____________

1 x F/C  100% 100        CR Coating          USD ____________

5.2.4 	FLAT BOTTOM		(Total area :4958 M2)

No. of coats  DFT       Product                Price

1 x T/U  30%  150        A/C Primer          USD

1 x T/U  30%  100        A/C Primer          USD

1 x F/C  100% 130        Antifouling SPC     USD _______________

5.3	Distinguishing Marks

Following marks to be painted with two full coats of paint, except where otherwise advised.

a.	3 sets of vessels name (on bows and stern)

b.	1 set port of registry (on stern)

c.	6 sets of draft marks  (fwd, midship, stern)

d.	2 sets of plimsoll marks (midship)

e.	42 sets of tank separating marks (port and stbd)

f.   2 sets of bulbous bow marks (on bow)

g.   6 sets of tugboat marks ( port and stbd )

h.   Cargo manifold marks ( 20 P and S)

i.   IMO no marking at stern

j.   Pilot Ladder (9M) marks P&S

i.	others if any

Cost of above lumpsump

including staging etc.				    USD

  - Tank cleaning

- Fuel oil/ Lub oil/ sludge USD           /M3

- Fuel oil DB			  USD           /M3

- Bilge water			  USD           /M3

- Fresh water			  USD           /M3

- Ballast tank	(Mud)		  USD           /M3

5.5  Ballast tank washing (SW)  USD           /tank

Removing/disposing of the wash water/sludge collected from above tank cleaning or tank washing should be on contractor’s account.

6. Cargo Hold Painting Work and services (Grain Standard) - Whole details are mentioned in Annexure 3-Attachement No. 04

7.0	Sea Chests

Remove all securings, lockings and grids of sea chests. Sea chests and their grids to be cleaned by scraping and high pressure jet. All grids and sea chests internal to be perfectly dry for examination by class surveyor and owner's superintendent. To apply 2 coats of A/F paint internally as specified for flat bottom. Refit the grids to sea chests using new locking split pins of stainless steel wire material SUS 30H WR, new stainless steel washers and nuts/ bolts as may be required.

Total 3 nos sea chests & locations and dimensions as shown

High sea chest : 1 no. size: 1600 X 800 mm (PORT)FR. 28-29 
Low sea chest  : 1 no. size: 1800x800 (STBD)FR. 29-30 
E/FIRE P/P S.C : 1 no. size, 640mm x 504mm (STBD)FR. 229-230

Quoted price to include all necessary staging, lighting etc.

Total                        			USD       _

8.	Bottom Plugs/ According to the plan

All ballast tank bottom plugs to be opened and re-closed. These plugs to be inspected by owner's superintendent after reclosing and prior to applying mastic putty. Lead gaskets to be renewed accordingly. Quoted cost to include removing the cement covering or weld strip locking the plug, and refitting / renewing same on completion of job, at least 12 hours prior to undocking.

Total no. of bottom plugs for Ballast tanks including forepeak tank and void spaces. 
1. F.P.TANK (C) 		:	1 
2. APT 		         	:	1 
3. APT ( P/S )        	:	0

4. C.W.T. (P/S)          :    0

5. B.T. ( C )            :

6. NO. 1 — 5, W.B.T(P/S) :    10

7. VOID SPACE            :    0

8. Other				:   6

Total:   18     PCS                  USD

9.	Corrosion Protection Anodes

Zinc anodes to be renewed on Water ballast tanksl, rudder and sea chests as required/instructed by owner's superintendent. Anodes renewal to be carried out prior to blasting jobs. Quoted cost to include cropping of existing wasted anodes as directed by the owner’s representative, as found necessary, and rewelding new anodes as directed.

Original anodes type Aluminium anodes,

BOLTED ANODES –

1. Water Ballast Tanks - 314 NOS.-By ship staff

TYPE : (90+110)MM X300 MM X100 MM

MATERIAL : Al-Zn-In

WEIGHT:  7.4 KG

( 5 year Life)

3. On the Rudder – 14Nos

TYPE  :   600 MMx150 MMx50 MM

MATERIAL : Al-Zn-In

WEIGHT   : 11.25Kg

( 5 year Life)

Ship’s hull is protected by a Cathodic ( ICCP) protection system.

Cost for supply and fitting of  ANODES        USD         per unit

Total cost for only removing & fitting of owner’s supplied anodes- Size to be checked

USD          per unit

Above quoted price to include all necessary staging, surface preparation etc.

WATER BALLAST TANK ANODES

314Nos  Al-Zn-In, with weight 7.4Kg (90+110x300x100mm)

W.B.T ZINC ANODES      QUANTITY

NO.1 W.B.T(P/S)            36

NO.2 W.B.T(P/S)            48

NO.3 W.B.T(P/S)            54

NO.4 W.B.T(P/S		       46

NO.5 W.B.T(P/S)            32

NO.1 TS W.B.T(P/S)         18

NO.2 TS W.B.T(P/S)         18

NO.3 TS W.B.T(P/S)         22

AP TK					  20

F.P.T 20

Cost for supply and fitting of  ANODES        USD         per unit

Total cost for only removing & fitting of owner’s supplied anodes

USD          per unit

10. Intermediate Shaft / Bearing & Tail shaft:  survey /withdrawal.

FOR QUOTATION AND REFERENCE PURPOSE ONLY. ONLY RENEWAL OF SEALS ARE PLANNED DURING THIS DOCKING. TAIL SHAFT WITHDRWAL WILL BE CARRIED OUT if required.

STERN TUBE        : 2350MM LENGTH ,DIA AFT-5350MM 
Seal type         : EVK2RV-472LR(WATER COOLED)

SEAL RING DWG. NO.: NA-002424800017

MAKER 		   : MICASA CORPORATION

ONE Intermediate Shaft Bearing — 445 mm Dia x 650 mm L,  
Tail shaft – LENGTH 5050 MM, DIA 420 MM

Tail shaft LINER : 430 mm Fwd/ 3505 mm aft
Intermediate shaft : 350	 mm dia x 6400 mm L

PROPELLER AND PROPELLER SHAFT(PROPELLER)

TYPE: 5 BLADES SOLID NBS TYPE,SKEWED (320) Right Hand

MATERIAL: NI-AL-BRONNZE Cu3

SIZE: DIA=6700 MM

PITCH: 5804.06MM

MAKER: NAKASHIMA PROPELLER CO.,LTD

Weight : 20395Kg

Propeller removal and refitting to replace AFT Shaft Sleeve if necessary ---  USD

(PROPELLER SHAFT) inspection

SIZE: D=420 MM X L=5,050MM X 1 SET

MATERIAL:KSF60

MAKER: SHIN KURUSHIMA DOCKYARD CO., LTD.

(INTERMEDIATE SHAFT)

SIZE:DIA=350MM X L=6,400 X 1 SET

MATERIAL :KSF65

MAKER: SHIN KURUSHIMA DOCKYARD CO., LTD

Intermediate bearing to be inspected --- USD

Drawings of stern tube seal, intermediate shaft and bearing and propeller shaft etc. attached

a.	Check the clearances of the stern tube bearings including check

including checking by feeler gauge and documentation of values

measured before dismantling.

- Propeller rope guard, cap and K-nut to be removed. Propeller to be removed as per maker’s specification and lowered on dock.

- Intermediate shaft bearing to be removed, inspect , measure wear.

- S

d.	Intermediate shaft coupling bolts on engine flywheel and tail shaft flange to be opened up and removed. Bolts to be marked before removal, clean and dye check before refitting.

e.	Intermediate shaft to be removed and kept aside. Associated piping if any, to be disconnected as required and blanked off if necessary.

f.	To remove the seal assy. from place, inspect condition of mating ring, clean / machine mating surface if necessary. To inspect /renew inflatable ring, if necessary. To remove spare seal fitted on tail shaft, inspect and assemble the seal assy. To fit a new seal on tail shaft as spares seal.

g.	Tail shaft to be withdrawn in the engine room and trueness to be checked.

h.	Shaft liner and Shaft taper, key and keyway  to be cleaned, inspect, magnaflux test to be carried out in presence of class surveyor/owner' superintendent.

i.	Tailshaft to be refitted in position. Seal assemblies to reinstall, and ‘run out’ to be checked and adjusted to be within maker’s specified limit.

j.	Shaft alignment to check and jack load test to be done to check load on intermediate shaft bearing and stern tube bearing.

k.	Propeller to be mounted and nut tightened and locked as per maker’s specifications/ instructions / push up graphs. Propeller cone to be refilled with yard supply tallow and to be refitted.

l.	Plummer block bearing and intermediate shaft to be repositioned, aligned, coupling bolts fitted and tightened. Bearing clearances to be recorded before opening and after fitting the shaft/ bearing and same to be recorded.

m. Any scoring/ cracks etc. in the shaft to be ground/machined, rewelded, NDT checked as per class requirements and remachined to original.

Quote for the above mentioned job inclusive of cleaning, staging and other necessary works which may include removal/ shifting and refitting of floor plates and their support angles, hand rails, sounding pipes etc, and preparation of area for hot work if found necessary.

a.	Disconnection and removal of intermediate

shaft, cleaning of plummer block, reassembling

of shaft/bearing and noting its clearances  USD

b.	Withdrawal of tailshaft, magnaflex test

and reinstallation.                         USD

c.	Removal and refit of propeller              USD

d.	Dismantling, overhauling, calibration,

reassembly of stern tube seal assy,

inflatable ring                             USD

e.   Cost of renewal of bearing forward and aft

stern tube bearing	                       USD

f.   Cost of repair by welding and machining.

Assume length of 1m for repair              USD

g.   Cost of checking the trueness of the

shaft                                       USD

Without removing the Propeller

h.	Quote for replacing owner supplied Stern seals insitu,

under OEM representative’s supervision      USD

With removing the Propeller

h.	Quote for replacing owner supplied Stern seals insitu,

under OEM representative’s supervision      USD

11.	Propeller

D04-03		PROPELLER SHAFT:

Propeller : Right Hand of Solid 5 Bladed keyless

Diameter : 6000 mm;

Material: Nickel, Aluminium, Bronze;(KAIBC3).

NAKASHIMA PROPELER.

Yard to furnish labour, materials, equipment, staging, carnage  and necessary work to accomplish the following:

A)	The cavitations damage / broken tips etc., to be repaired as found necessary. Repair procedure to be submitted to the owner for review and subsequent agreement with the class DNV, post repair NDT, if any, strictly upon pre-approval from the owner.

Quote Price

B)	Quote Price for propeller buffing / polishing

Quote Price:   USD …………

C)	Propeller boss to dye check for cracks in way of blade roots and tips after polishing has been carried out .

Quote Price:

D) To measure Propeller shaft wear down in the presence of ship chief engineer. All readings to be submitted to attending superintendent immediately on completion. To drain the oil,  to take the poker gauge reading,  to refill the EAL oil and finally to carry out the tightness test in place

Quote Price

TYPE: 5 BLADES SOLID NBS TYPE,SKEWED (320) Right Hand

MATERIAL: NI-AL-BRONNZE Cu3

SIZE: DIA=6700 MM

PITCH: 5804.06MM

MAKER: NAKASHIMA PROPELLER CO.,LTD

Weight : 20395Kg

ALL JOBS BELOW TO BE QUOTED. JOB SCOPE TO BE DECIDED AFTER EXAMINATION.

Propeller to be examined and surveyed. Propeller to be polished on all blades by power sanding, 3R part to be examined by dyepenetrant, inspected by class surveyor. Cavitation pits/ damaged portion if  found on the blades to be repaired by brazing and ground off to original profile . Cost of staging, temporary lighting etc to be included in the prices quoted below.

a.	Quote cost for repairing eroded pits

by brazing and ground off to flat

for estimation: dia 30 x 2mm dept x 30 pits

Material: NI-AL-Br	CU3			  USD

b.	Quote cost for

Normal polish                         USD

Super polishing to rupert scale A     USD

c.	Cost for carrying out Non destructive

test on blades.

USD

d.	Cost of underwater polishing          USD _________________

12.	Propeller Shaft

Using the same staging for polishing the propeller, remove rope guards. To measure the  stern bush wear down readings in attendance of Chief Engineer. Rope guard to be refitted by intermittent welding and net cutter to be furnished around the rope guard. Record of wear down readings to be submitted to class surveyor and owner's superintendent, soon after measurement. Anode to be fitted with yard supply.

a. Cost for removing / refitting rope guard   USD

b. Cost for calibration.                      USD

c. Price to weld 4 pcs net cutter around

refitted rope guard.

Net cutter to yards supply                 USD

d. Price for fitting the yard supply anodes   USD _____________

13.	Rudder

Type of rudder     : Streamline semi balanced 
Rudder upper and lower sleeve: Bronze

Rudder neck bearing bush: Bronze

Rudder pintle bush: PHENOL RESIN

Rudder area : 19.22 M2

Balancing ratio: 0.306

Aspect ratio: 2

Rudder body: 7199 kg

Rudder stock: 4915 kg

### SCOPE OF JOB TO BE DECIDED UPON INSPECTION

Rudder to be inspected visually for damage. Rudder wear down, top and bottom pintle clearances to calibrate per class requirement and record submitted to class surveyor and owners superintendent. Quoted costs to include all staging and lighting as necessary.

a.	Quote cost of carrying out calibration .

Rudder wear down, top and bottom pintle

bush clearance, inclusive of removal and

rewelding of pintle inspection

plate as necessary                        USD

b.	Cost of removing/rewelding inspection

plate (per piece ) to remove the plate

for inspection, carry out hardening of

pintle nut and clearances and rewelded

in place upon inspection by owners  	USD

c.	Price per M rebuilding up wasted welding

seams after gauging and veeing.        	USD

d.	Removing and refitting of rudder drain

plugs with new gaskets ( 2 nos)         USD

e.	Pressure testing with air/soap solution

(0.5 bar) each time                     USD          /time

f.	Repacking transom space gland packing

(packing yard supply)                   USD

g.	Cavitation pits to be filled by welding

for estimation assume a pit is 100 MM

in dia and 5Mm dept.                    USD

14.	Rudder Top and Bottom Pintle bushes

On reviewing the clearances at the top neck bearing and bottom pintle, if found in excess or near limits, and if renewal of the bushes is decided upon, quote for necessary work involved in the removal and refitting of the rudder, opening, removing and refitting of the pintles nuts/ pins and tightening to original specifications, removal of worn pintle bushes and renewal/ refitting of new bushes [yard supply].

a.	Chip off and remove cement from palm bolt

flange between rudder/rudder stock and

re-cement same [with yard supply cement]

upon completion.                            USD

b.	Dismantling and refitting of palm bolts,

pintle nut and other fastenings as required for the

job, and refitting of same on completion    USD

c.	Labour, equipment and crane service

to dismantle, unshipping, shipping

and re-fitting of the rudder as required    USD

d.	To calibrate the pintles, sleeves and liner

at both  spaces			        		   USD

e.	To remove upper stock bush and lower pintle

bushes and fit new bushes [yard supply] which

have been machined/ made to required

specifications as necessary. Locking/ keep

plates to be removed and refitted as necc   USD

f.	To carry out magnaflux test in way of sleeve

areas for detection of cracks               USD

g.	To calibrate and record all pintle/ bush

clearances before and after the renewal of

the phenol resin / bronze bushes		   USD

i.	Weld build up on bush housing and

boring to required size including

key stoppers if needed.                 	   USD

j.	To machine upper and lower shaft bushes

with yard supplied material             	   USD

k.	Alignment check after repairs               USD

l.	To carry out serving test to the satisfaction

of owner's representative and class surveyor

upon job completion                         USD

m.   To remove the lipseal from the rudder stock

and renew with new seal and O-ring          USD _______________

n.   Jumping collar/bar to check, replace if required

and record clearance.                       USD _______________

o.   Examine rudder pintle castings for cracks    USD _______________

p.   Repair of pintle casting cracks if found    USD _______________

Above mentioned costs to include all staging, supports, lifting tackle, temporary lighting, cranage, ventilation if necessary, opening and refitting of pintle inspection/ access plates as required, testing/ inspections at various stages to class surveyor’s requirements.

15.	Anchors and cables

Anchor cables port/stbd side to be ranged out on dock bottom for cleaning by HP fresh water jet of minimum 400 bar and inspection [allow for manual turning of relevant sections of chain to gain access to undersides for cleaning/ access]. Anchor shackle pins, swivels, kenter shackles link studs to be examined by owner's superintendent. Anchors and cables to be sprayed with bituminous paint, kenter shackles to be marked by hemp cloth and secured by stainless band and painted with red and white paint before restoring

Forward Anchor (2 Nos): 9900 kgs (Each). stockless anchor 
Port side chain 78 mm dia x 330.00 M, GRADE G3 (a) chain steel
Stbd side chain 78 mm dia X 330.00 M, GRADE G3(a) chain steel

a.	Quote cost for lowering using

yard cranes if necessary /ranging out/

restoring anchor and its cable. Cost

to include opening and refitting of

bitter end if found necessary        	USD          /2 sets

b.	Quote cost for cleaning by HP fresh

water wash of minimum 400 bar     		USD           /2sets

c.	Cables to be calibrated and recorded .

Quote cost for calibration        		USD           /2sets

d.	Quote cost for applying bitumastic coat

over anchors and cables. Coating product

to be supplied by yard            		USD           /2sets

e.	Price for marking over shackles as

indicated earlier. Coating/ marking

products and material to be supplied

by the yard. 				    		USD           /2 sets

f.	Price per pc rewelding slacked link

stud (one side only)              		USD              /pc

g.	Price per  new piece

Kenter shackle pin                		USD              /pc

D-Shackle pin

USD              /pc

Kenter shackle		USD              /pc

Chain Link Shackle USD              /pc

D-Shackle

USD              /pc

h.	Repair of anchor crown/shank pin by

building up and machining inclusive

of dismantling, reassembling and

transportation charges to and from 		USD          /anchor

i.	Shifting first chain shackle on the

place of last one and vice versa. 		USD         /shackle

j.	Renewal of missing lead pellets

k.   To install new anchor and chain to the

stbd chain locker and mark same          USD __________________

16.	Chain Locker

Both chain lockers to clean and remove all water and mud from their bottoms . One full coat of bitumastic solution to be applied to the entire internal surface of both lockers. Resecuring of shackles in the lockers to be checked by Chief officer.

Size =1.8m x 2.1MX 6.6M/each

### QUOTE FOR RECOATING TO BE DONE UPTO 3M FROM BOTTOM

a.	Price per M3, pumping out/disposal

of remaining water                 		USD               /M3

b.	Price per MT. collecting and disposal

of mud from the lockers            		USD               /MT

c.	Price for scraping, cleaning and applying

full coat of bitumastic solution over both

lockers interior (owners supply coating)	USD          / LOCKER

17. Hull Thickness Gauging

Quote cost for carrying out random SPOT thickness gauging of various areas of hull plating, deck plating, tank/hold bulkheads and structures as required in DNV-GL Class Renewal Survey No 1.

For Hull Plating charge per point.          / point

For Deck Plating charge per point.          / point

For Cargo hold/ Bulk heads charge per point.          / point

For DB Tanks/void spaces charge per point.          / point

For structural members charge per point.          / point

For larger diameter pipes charge per point.          / point

18. Overhaul of Governors

Quote cost for overhaul of auxiliary engine governors, inclusive of disconnecting and removal from site, opening and overhaul, replacement of worn parts , reassembly and test run to satisfaction, refitting in place and operational test. Running test on engine also to be carried out at no costs to the owners. Work to be carried out by makers authorized service centre.

AUX. ENGINE GOVERNOR-3NOS.

AUX ENGINE MAKE:DAIHATSU

TYPE:6 DK 20e

GOVRNOR:

Make:ZEXEL

TYPE:- RHD 6

Please quote per governor. USD

19. Main Switch-board Overhaul and Cleaning

Quote cost for carrying out the cleaning and overhaul/ tightening/ dry-dock routine maintenance of the Main Switch-boards in the engine control room, and all associated auxiliary distribution pannels in the engine room.

Cost for maintenance of the ECR MAIN switchboards  	USD

Cost for maintenance of the Emergency switchboard  	USD

Cost for maintenance of auxiliary distribution panels	USD

Above mentioned job to be carried out at night [with no extra overtime compensation] and to include provision of temporary lighting and ventilation at work areas and access ways as required, and cost for same to be included in the quote.

20. Repairs / Renewal of Shipside Frames and Brackets

Quote the price per/kg applicable for the renewal of sections of shipside frames in water ballast tanks which may be found damaged. Cost to include the cropping and removal of the damaged sections of frames, surface preparation, fabrication and welding of suitable replacement sections where necessary, and subsequent surface preparation and painting of affected area of shipside exterior with owner’s supplied paint (frame and shipside interior will be painted by vessel’s staff. The unit price will be applicable for the actual weight of steel renewed in these small sections, and no further ‘wastage factor’ will be applicable.

Cost per Kg  USD

21. Rewinding of Electric Motors

Kindly advise the cost for

a)removal, dismantling, rewinding (including varnishing and baking), reassembly with new bearings (including dressing up of shaft seatings and bearing housing areas if found necessary), bench test on completion of work, and return to the vessel. Advise cost for motors of following rating: 1kw, 3kw, 5kw, 10kw, 15kw, 20kw, 30kw, 40kw, 60kw.

b) removal, dismantling, reassembly with new bearings (including dressing up of shaft seatings and bearing housing areas if found necessary), bench test on completion of work, and return to the vessel

Advise cost for motors of following rating: 1kw, 3kw, 5kw, 10kw, 15kw, 20kw, 30kw, 40kw, 60kw. ------------USD

22.	Ship Side Valves

To open, clean, lap and overhaul as required, and present for inspection to surveyor’s and superintendent’s satisfaction, and to box back after completion. Price to include replacement of damaged nuts/bolts, renewal of joints and gland packing, cleaning and lapping of valve & seat, etc, cleaning of the valve body and thereafter painting the interior with one coat of yard’s supplied apexior. Also quote for supply and fitting of new valve if found necessary.

List of valves to be overhauled is attached.

Quote cost for overhaul of individual valves and also lumpsum for overhaul of all valves mentioned Below.

23. Boiler

Oilfired and smoke side tube to be chemically cleaned.
Water side to be cleaned.
Main steam pipe line section removed pressure tested in the presence of DNV surveyor and stamped by class

Mounting Overhaul

Make:SAACKE GMBH

Südweststraße 13 28237 Bremen

e-mail : internet :info@saacke.de

www.saacke.com

24. Ship Side Valves   - More details

|  | Valve | DN | Valve Type | Name on plate | Location | Dimentions on Drawing (MPa) | Dimentions from local check (bar) | Cost for overhaul as detailed above and survey | Cost for removing valve to workshop and refit after skimming of valve lid and seat | Cost for open, remove refit new valve owners supply | Cost for open, remove, refit new valve of yard supply |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Overboard Discharge Valves |  |  |  |  |  |  |  |  |  |  |
| 1 | Ballast Pump 1 to OVBD | DN350 | Butterfly V. | BMV09 | E/R | 1DN350 | 10DN350 |  |  |  |  |
| 2 | Ballast Pump 2 to OVBD | DN350 | Butterfly V. | BMV10 | E/R | 1DN351 | 10DN350 |  |  |  |  |
| 3 | Ballast Strip. Eductor Discharge OVBD | DN150 | Stop Check v. | BMV16 | E/R | 0.6DN150 | 6DN150 |  |  |  |  |
| 4 | Bilge Eductor for C/H Discharge OVBD | DN125 | Stop Check v. | BGV27 | E/R | 0.6DN125 | 6DN125 |  |  |  |  |
| 5 | Main Bilge Pump to OVBD | DN250 | Stop Check v. | BGV13 | E/R | 0.6DN250 | 6DN250 |  |  |  |  |
| 6 | Oily Bilge Water Separator to OVBD | DN40 | Stop Check v. | OWV04 | E/R | 0.6DN50 | 25DN50 |  |  |  |  |
| 7 | Sewage Discharge OVBD | DN150 | Storm Vv | SWV01 | E/R | 0.6DN150 | 6DN150 |  |  |  |  |
| 8 | Ref. chamber Discharge OVBD | DN100 | Stop Check v. | GPV01 | E/R | 2.5DN100 | 25DN100 |  |  |  |  |
| 9 | Sea Water Cooling Pump to OVBD | DN300 | Stop Check v. | CSV19 | E/R | 0.5DN300 | 18DN300 |  |  |  |  |
| 10 | Fresh water Generator Sea Water outlet OVBD | DN80 | Stop Check v. | CSV20 | E/R | 0.5DN80 | 5DN80 |  |  |  |  |
| 11 | Boiler Blow to OVBD | DN40 | Boiler Side Drain V. | FEV08 | E/R | 2.5DN32 | 10DN40 |  |  |  |  |
| 13 | BWTS Back Flushing to OVBD | DN125 | Butterfly V. | BMV34 | E/R | 1DN125 | 10DN125 |  |  |  |  |
| 14 | F.P. TK. Ballast water Overflow to OVBD (P) | DN300 | Stop Check v. | AVV101 | Fwd below Boatswain store | 0.6DN300 | 18DN300 |  |  |  |  |
| 15 | F.P.TK. Ballast Water Overflow to OVBD (S) | DN300 | Stop Check v. | AVV102 | Fwd below Boatswain store | 0.6DN300 | 6DN300 |  |  |  |  |
| 16 | AFT. Mooring winch Cooling Water to OVBD | DN50 | Stop Check v. | FFV244 | S.G Room |  | 25DN50 |  |  |  |  |
| 17 | Bilge Eductor for C/L Discharge OVBD | DN80 | Stop Check v. | BGV130 | Fwd below Boatswain store | 0.6DN80 | 3DN80 |  |  |  |  |
|  | Sea Suction Valves |  |  |  |  |  |  |  |  |  |  |
| 18 | Low Sea Suction | DN500 | Remote Controlled Butterfly Valve Flanged | CSV24 | E/R | 1DN500 | 10DN500 |  |  |  |  |
| 19 | Low Sea Chest Air Vent | DN50 | Stop Valve(Bronze) | CSV22 | E/R | 2.5DN50 | 25DN50 |  |  |  |  |
| 20 | Low Sea Chest Steam blow |  | Stop Valve | STV24 | E/R | 2.5DN15 | 25DN20 |  |  |  |  |
| 21 | From Central FW Cooler | DN200 | Stop Check Valve | CSV21 | E/R | 0.6DN200 | 6DN200 |  |  |  |  |
| 22 | High Sea Suction | DN500 | Remote Controlled Butterfly Valve Flanged | CSV25 | E/R | 1DN500 | 10DN500 |  |  |  |  |
| 23 | High Sea Chest Air Vent | DN50 | Stop Valve(Bronze) | CSV23 | E/R | 2.5DN50 | 25DN50 |  |  |  |  |
| 24 | High Sea Chest Steam blow |  | Stop Valve | STV25 | E/R | 2.5DN15 | 25DN20 |  |  |  |  |
| 25 | Emergency F.P Suction Valve | DN125 |  | EFV01 | E.F.P Room | 0.4DN125 | 4DN125 |  |  |  |  |
| 26 | E.F.P Sea suction Air pipe | DN65 | Stop Valve | EFV03 | E.F.P Room | 2.5DN65 | 25DN65 |  |  |  |  |
| 27 | Compressed air blow for sea chest | DN20 | Stop Valve | EFV04 | E.F.P Room | 2.5DN20 | 25DN20 |  |  |  |  |
| 28 | Aft Draft Sensor Inlet | DN40 | Globe valve |  | E/R | DN40 | 25DN40 |  |  |  |  |
| 29 | Mid Draft (P) Sensor inlet | DN65 | Stop Valve |  | pipe tunnel | DN65 | 16DN25 |  |  |  |  |
| 30 | Mid Draft (S) Sensor inlet | DN65 | Stop Valve |  | pipe tunnel | DN65 | 16DN25 |  |  |  |  |
| 31 | FWD Draft Sensor inlet | DN25 | Globe valve |  | E.F.P Room | DN25 | 25DN25 |  |  |  |  |

25. Schedule Jobs under  Annexure 2

mentioned job lists, are seperately attached with full job specification. If any clarification needed, contact Technical Superintendent – ts@cscl.lk,dpa@cscl.lk

As per the attached job list in Annexure 02.

- Deck side jobs-  50 jobs

- Engine Side jobs -38 jobs

- Electrical jobs -16 jobs

All jobs to be quoted.